Bryant Fire Dept.

www.cityofbryant.com

SCBA

BID SPECIFICATIONS

Bid shall be for two options, (A) and (B):

Option (A):

(25)-  SCBA 2007 NFPA Edition units each containing:

Intergraded PASS

Quick Connect Regulator

Dual Emergency Breathing Support System

Internal Tracking Capability (for downed firefighter location)

(1) Face Piece with Kevlar Head Harness

(1) 45 Minute Carbon Composite Cylinder

(25)- Additional 45 Minute Carbon Composite Cylinders

(28)- Additional face pieces

(1)- Handheld tracking receiver

Any tools, training, software and upgrades to existing equipment necessary for the department to perform routine SCBA maintenance

Applicable sales tax and shipping

Option (B):

(40)-  SCBA 2007 NFPA Edition units each containing:

Intergraded PASS

Quick Connect Regulator

Dual Emergency Breathing Support System

Internal Tracking Capability (for downed firefighter location)

(1) Face Piece with Kevlar Head Harness

(1) 45 Minute Carbon Composite Cylinder

(40)- Additional 45 Minute Carbon Composite Cylinders

(13)- Additional face pieces

(1)- Handheld tracking receiver

Any tools, training, software and upgrades to existing equipment necessary for the department to perform routine SCBA maintenance

Applicable sales tax and shipping

NOTE: Currently the Department has a PosiCheck 3 machine with ISI software and a serial type data port.

Any questions should be directed to:

J.P. Jordan

Fire Chief

501-943-0390

jpjordan@cityofbryant.com

GENERAL  REQUIREMENTS

The purpose of this bid specification is to establish the minimum requirements for an open-circuit self-contained breathing apparatus (SCBA). The SCBA shall consist of the following major sub-assemblies: (1) full facepiece assembly; (2) a removable, facepiece-mounted, positive pressure breathing regulator with air-saver switch; (3) an automatic dual path redundant pressure reducing regulator; (4) end-of-service time indicators; (5) a harness and backframe assembly for supporting the equipment on the body of the wearer; (6) a shoulder strap mounted, remote gauge indicating cylinder pressure; (7) a rapid intervention crew/universal air connection (RIC/UAC); and (8) cylinder and valve assembly for storing breathing air under pressure.

The successful bidder agrees to provide, at their own expense, a factory trained instructor for such time as the respirator user shall require complete instruction in the operation and maintenance of the respirator. Any exceptions to these specifications must be detailed in a separate attachment. Failure to do so will automatically disqualify the bidder.

The successful bidder must be a sales distributor, authorized by the manufacturer, to sell the equipment specified herein and have an in-state authorized service center.  A signed document from the manufacture confirming this must be included with the bid.

The SCBA shall maintain all MIOSH standards with any of the following types of cylinders listed as provided by the SCBA manufacturer.

|  | Product: |  |  |
| --- | --- | --- | --- |
| Approvals | Meets | Does Not Meet | Exception |
| *The SCBA shall be approved to NIOSH 42 CFR, Part 84 for chemical, biological, radiological and nuclear protection (CBRN). |  |  |  |
| *The SCBA shall be compliant to the NFPA 1981, 2007 Edition, Standard on Open-Circuit Self-Contained Breathing Apparatus for Emergency Services. |  |  |  |
| *The SCBA shall be compliant to the NFPA 1982, 2007 Edition (if including optional PASS Device), Standard on Personal Alert Safety Systems. |  |  |  |
| *If the SCBA is to include an optional integrated self- Rescue device, the device shall be compliant to the NFPA 1983, 2006 Edition, Standard on Life Safety Rope and Equipment for Emergency Services. |  |  |  |
| *All electronic components shall be approved for Intrinsic Safety under UL913 Class 1, Groups C and D, Class II, Groups E, F and G, Hazardous locations. |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| Mask-Mounted Regulator | Meets | Does Not Meet | Exception |
| *The facepiece-mounted positive pressure-breathing regulator shall supply and maintain air to the facepiece to satisfy the needs of the user at a pressure greater than atmospheric by no more than 1.5 inches of water pressure static. |  |  |  |
| *The breathing regulator shall maintain positive pressure during flows of up to 500 standard liters per minute. |  |  |  |
| *The breathing regulator shall have attached a low pressure hose which shall be threaded through the left shoulder strap to couple to the pressure reducing regulator mounted on the backframe. |  |  |  |
| *The regulator shall be equipped with a quick connect coupling in line for use with the optional outlet manifold and accessory hose to allow the breathing regulator to be disconnected from the unit and reconnected to the auxiliary hose of a second unit in the even rescue is required. |  |  |  |
| *The quick connect coupling shall be easily connected and disconnected by trained individuals with a gloved hand and/or in low light conditions. |  |  |  |
| *The coupling shall also be guarded against inadvertent disconnect during use of the equipment. |  |  |  |
| *The low-pressure hose shall be equipped with a swivel attachment at the facepiece mounted regulator. |  |  |  |
| * The regulator shall connect to the facepiece by way of a quarter (1/4) turn coupling. |  |  |  |
| *The user shall hear an audible sound when the regulator is attached correctly to the facepiece. |  |  |  |
| *The regulator shall be equipped with a doughnut-shaped gasket which provides a seal against the mating surface of the facepiece. |  |  |  |
| * The regulator cover shall be fabricated of a flame resistant, high impact plastic. |  |  |  |
| *The breathing regulator shall have a demand valve to deliver air to the user, activated by a diaphragm responsive to respiration. |  |  |  |
| *The demand valve shall use an extended temperature range dynamic O-ring seal composted of a fluorosilicone elastomer. |  |  |  |
| * The diaphragm shall include the system exhalation valve and shall be constructed from a high strength butyl elastomer. |  |  |  |
| *A purge valve shall be situated at the inlet of the breathing regulator and shall be capable of delivering airflow of between 125 and 175 standard liters per minute. |  |  |  |
| *The breathing regulator shall be arranged to direct the incoming air over the inner surface of the facepiece for defogging purposes. |  |  |  |
| * The components of the breathing regulator shall be constructed of materials that are not vulnerable to corrosion. |  |  |  |
| *The flame resistant cover shall contain an air saver switch and pressure demand bias mechanism. |  |  |  |
| *It shall reactive and supply air only in the positive pressure mode when the wearer affects a face seal and inhales. |  |  |  |
| *This device shall not affect the breathing flow through the system while in operation. |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| Pressure Reducer | Meets | Does Not Meet | Exception |
| *The pressure-reducing regulator shall be mounted on the backframe and be coupled to the cylinder valve through a short length of internally armored high pressure hose with a hand coupling for engagement and sealing within the cylinder valve outlet. |  |  |  |
| *In lieu of a manual by-pass, the pressure-reducing regulator shall include a back-up pressure-reducing valve connected in parallel with the primary pressure reducing valve and an automatic transfer valve for redundant control. |  |  |  |
| *The back-up pressure reducing valve shall also be the means of activating the low-pressure alarm devices in the facepiece-mounted breathing regulator. |  |  |  |
| *This warning shall denote a switch from the primary reducing valve to the back-up reducing valve whether from a malfunction of the primary reducing valve or from low cylinder supply pressure. |  |  |  |
| *A press-to-test valve shall be included to allow bench testing of the back-up reducing valve. |  |  |  |
| *The pressure-reducing regulator shall have extended temperature range dynamic O-ring seals composed of fluorosilicone elastomer. |  |  |  |
| *The pressure reducing regulator shall have incorporated a reseatable over-pressurization relief valve which shall prevent the attached low pressure hose and facepiece-mounted breathing regulator from being subjected to high pressure. |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| End-of-Service Time Indicator (EOSTI) | Meets | Does Not Meet | Exception |
| *The SCBA shall have two (2) end-of service indicators (EOSTI). A tactile alarm and a Heads-Up Display (HUD). |  |  |  |
| *The primary EOSTI shall be the integral low-pressure alarm device that shall combine an audible alarm with simultaneous vibration of the facepiece. |  |  |  |
| *The primary EOSTI shall be located in the Facepiece-Mounted Positive Pressure Regulator. |  |  |  |
| *This alarm device shall indicate either low cylinder pressure (less than 25%) or primary first stage regulator failure. |  |  |  |
| *The HUD shall serve as the secondary EOSTI indicator. |  |  |  |
| *It shall be mounted in the user’s field of vision on the second stage regulator. |  |  |  |
| *It shall display one-quarter bottle increments including full bottle pressure and continuing to 25% of maximum bottle pressure. |  |  |  |
| *The display shall not have a numerical representation of bottle pressure. |  |  |  |
| *At full bottle pressure, two (2) green Light Emitting Diodes (LED) shall be illuminated. |  |  |  |
| *At three-quarter bottle pressure, one (1) green LED shall be illuminated. |  |  |  |
| *At one-half bottle pressure, one (1) yellow LED shall be illuminated and flash at a rate not to exceed one (1x) time per second. |  |  |  |
| *At one-quarter bottle pressure, one (2) red LED shall be illuminated and flash at a rate not to exceed 10 (10X) times per second. |  |  |  |
| *The HUD shall have a low battery indication that is distinct and distinguishable from the bottle pressure indications. |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| Harness and Backframe Assembly | Meets | Does Not Meet | Exception |
| *A lightweight, lumbar support style backframe and harness assembly shall be used to carry the cylinder and valve assembly and the pressure reducing regulator assembly. |  |  |  |
| *The backframe shall be a solid, one-piece black powder-coated aluminum frame that is contoured to follow the shape of the user’s back. |  |  |  |
| *The backframe shall include a mounting for the pressure reducer. |  |  |  |
| * This mounting shall contain a slide-type bracket permitting positioning of the pressure reducer to accommodate connection to either an angled or straight-type cylinder valve. |  |  |  |
| *The backframe shall include an over-the-center adjustable tri-slide fixture, a para-aramid strap and a double-locking latch assembly to secure a 45 minute cylinders. |  |  |  |
| *The harness assembly shall consist of a one-size black para-aramid strap with a yellow stripe. |  |  |  |
| * This harness shall include box-stitched construction with no screws or bolts. |  |  |  |
| * The harness assembly shall incorporate parachute-type, quick-release buckles and shall include shoulder and hip pads. |  |  |  |
| * The harness shall include a seat-belt type waist attachment. |  |  |  |
| * The shoulder strap shall be fitted with a Drag Rescue Loops (DRL) capable of being deployed in an emergency situation to drag a downed firefighter to safety. |  |  |  |
| * The one-piece aluminum backframe should include integrated donning/carry handles. |  |  |  |
| * The handles shall allow the user to easily don the SCBA in the “over-head” style and also allow the user to carry the SCBA. |  |  |  |
| * The backframe shall include accommodation and mounting spaces suitable for installation of a distress alarm integrated with the SCBA. |  |  |  |
| * These mounting spaces shall permit installation of an alarm sensor module in an area between the cylinder hanger locking mechanism and the backframe. |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| Shoulder-Mounted Pressure Gauge | Meets | Does Not Meet | Exception |
| * The pressure gauge shall be an integral part of the control console assembly. |  |  |  |
| * The control console shall come with a mechanical (analog) pressure gauge that is angled at 30°. |  |  |  |
| * The control console shall contain an edge lit pressure gauge that requires no action by the user to turn on except open the cylinder valve. |  |  |  |
| * The control console shall contain a photo sensing diode to dim and brighten the HUD as the environment changes. |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| Rapid Intervention Connection | Meets | Does Not Meet | Exception |
| * The SCBA shall incorporate a RIC/UAC fitting to be compliant with the 2007 edition of the NFPA 1981 Self-Contained Breathing Apparatus standard. |  |  |  |
| * The RIC/UAC shall be an integral part of the high-pressure hose that attaches the cylinder valve to the first stage pressure reducer. |  |  |  |
| * The RIC/UAC inlet connection shall be within 4” (4-inches) of the tip of the CGA threads of the cylinder valve. |  |  |  |
| * The RIC/UAC shall consist of a connection for attaching a high-pressure air source and a self-resetting relief valve allowing a higher pressure than that of the SCBA to be attached to the SCBA. |  |  |  |
| * The RIC/UAC shall have a check valve to prevent the loss of air when the high-pressure air source has been disconnected. |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| Cylinder | Meets | Does Not Meet | Exception |
| * The cylinder threads shall be straight with an O-ring or quad-ring gasket type seal. |  |  |  |
| * The cylinder valve shall be a “fail open” type, constructed of forged aluminum and designed such that no stem packing or packing gland nuts are required. |  |  |  |
| * It shall contain an upper and lower seat such that the pressure will seal the stem on the upperseat, thus preventing leakage past the stem. |  |  |  |
| * No adjustment shall be necessary during the life of the valve. |  |  |  |
| * The cylinder valve outlet shall be a modification of the Compressed Gas Association (CGA) standard threaded connection number CGA 347 for 4500 systems with a tri-lobe ergonomically designed handwheel. |  |  |  |
| * The valve shall be constructed such that damage will not occur if the coupling is over-torqued by hand. |  |  |  |
| * Each cylinder valve shall consist of the following: (1) a hand activated valve mechanism with a spring-loaded, positive action, ratchet type safety lock and lock-out release for selecting “lock open service” or “non-lock open service”; (2) an upstream connected fangible disc safety relief device; (3) a dual reading pressure gauge indicating cylinder pressure at all times; (4) an elastomeric bumper; (5) an angled outlet. |  |  |  |
| * Each cylinder and valve assembly shall be equipped with a hanger bracket for positive locking attachment of the assembly to the backframe. |  |  |  |
| * The SCBA shall maintain all NIOSH and NFPA standards with any of the following types of cylinders listed as provided by the SCBA manufacturer. |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| Carbon-Wrapped | Meets | Does Not Meet | Exception |
| * The cylinder shall be manufactured in accordance with DOT specifications and meet the Transport Canada requirements with working pressure of 4500 psig. |  |  |  |
| * The cylinder shall be lightweight, composite type cylinder consisting of an aluminum alloy inner shell, with a total overwrap of carbon fiber, fiberglass and an epoxy resin. |  |  |  |
| * The cylinder shall be available in a 45-minute duration based on the NIOSH breathing rate of 40 liters per minute (1pm). |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| Warranty | Meets | Does Not Meet | Exception |
| * The unit shall be covered by a warranty providing protection against defects in materials or workmanship. |  |  |  |
| * This warranty shall be for a period of ten (10) years on the SCBA, except for the pressure reducer, which shall be covered for fifteen (15) years. |  |  |  |
| * Electronic components shall be warranted for three (3) years. |  |  |  |

|  | Product: |  |  |
| --- | --- | --- | --- |
| Emergency Breathing Support System “Buddy Breathing” | Meets | Does Not Meet | Exception |
| * The Dual Emergency Breathing Support System (EBSS) shall have one of each of the following requirements: (1) a manifold with one each of a female socket and male plug, both of which have check valves; (2) 40” minimum low-pressure hose; (3) a pouch for storing hose; and (4) a dust cap for the female socket and male plug. |  |  |  |
| * The Dual EBSS system shall be on the wearer’s left side and shall be capable of allowing for six (6) feet of hose between like systems. |  |  |  |
| * The manifold shall be made of aluminum and be anodized black. |  |  |  |
| * The female socket and male plug shall have spacing, no less that 15° off-center. |  |  |  |
| * The female socket shall have a double action to disengage, noted as a “push-in/pull-back”. |  |  |  |
| * The female socket shall have an internal check valve. |  |  |  |
| * The male plug shall have an external check valve. |  |  |  |
| * The hose shall be made of high temperature rubber capable of sustaining a maximum 250 psig of pressure. |  |  |  |
| * The containment system shall include a pouch and shall be made of para-aramid materials and shall be capable of storing 36” of hose. |  |  |  |
| * The pouch shall be attached to the SCBA by pull-the-dot-fasteners. |  |  |  |

| Extended Duration Airline System | Meets | Does Not Meet | Exception |
| --- | --- | --- | --- |
| * An optional manifold shall also have provision for connection of an airline supply for extended duration use while reserving the cylinder supply for egress. |  |  |  |
| * The air supply hose length shall be up to 300 feet and require an inlet pressure range of 60 to 115 psig, depending on the length of supply hose used. |  |  |  |
| * The check valve within the outlet manifold shall prevent the external release of cylinder air in the even the air supply is either not used or disconnected. |  |  |  |
| * Switching from airline supply to cylinder supply shall be accomplished manually by the user by opening the cylinder valve to prevent inadvertent use of the cylinder supply without the user’s knowledge. |  |  |  |